Reference Software for View Synthesis

Version 2.3

June 27th 2008

Kazuyoshi SUZUKI

Nagoya University, Japan

- Software specification

This software generates virtual view image by using two types of depth value: (Type I) depth from camera, (Type II) depth from the origin of 3D space. Type selection is as follows.

- view synthesis by using depth value from camera  Type I

- view synthesis by using depth value from the origin of 3D space  Type II

Type I method convert the input depth values from camera into depth values from the origin of 3D space by using z component of translation vector of camera. Akko&Kayo and Rena sequences of Nagoya Univ. have to be estimated by Type I method because depth values from the origin of 3D space take not only positive values but also negative values. On the other hand, Champagne_tower, Dog, and Pantomime sequences of Nagoya Univ. and Microsoft Research sequences after rectification are estimated depths by Type II method because all depth values take positive values. In this software, Type I method is applicable to only the case that the direction of camera is opposite to the direction of z axis of 3D space. The difference between Type I method and Type II method is explained in Appendix.

  - Input/output file format

In the previous version of this software, YUV 4:2:0 files are inputted or outputted frame by frame. In the new version, YUV 4:2:0 video files are inputted or outputted.

  - Input parameters

- DepthType

Flag 0 or 1.

Description: Flag 0 selects the mode which generates virtual view by using depth from camera. Input depth values are automatically transformed into depth values from the origin of 3D space by referencing z component of camera translation vector. Flag 1 selects the mode which generates virtual view by using depth from the origin of 3D space.

- WidthOfViewImage

Class: Integer

Description: Width of view image.

- HeightOfViewImage

Class: Integer

Description: Height of view image.

- NearestDepthValueOfLeftView

Class: Double

Description: Nearest depth value Znear of left view from the origin of 3D space or camera.

- FarthestDepthValueOfLeftView

Class: Double

Description: Farthest depth value Zfar of left view from the origin of 3D space or camera.

- NearestDepthValueOfRightView

Class: Double

Description: Nearest depth value Znear of right view from the origin of 3D space or camera.

- FarthestDepthValueOfRightView

Class: Double

Description: Farthest depth value Zfar of right view from the origin of 3D space or camera.

- TotalNumberOfFrames

Class: Integer

Description: Total number of frames which are generated virtual views.

- LeftCameraName

Class: String

Description: A name of left real camera.

- VirtualCameraName

Class: String

Description: A name of virtual camera.

- RightCameraName

Class: String

Description: A name of right real camera.

- LeftViewImageName

Class: String

Description: A name of left image. File format is YUV 4:2:0. File name has to be the form that “LeftViewImageName.yuv”. Name may include file path.

- RightViewImageName

Class: String

Description: A name of right image. File format is YUV 4:2:0. File name has to be the form that “RightViewImageName.yuv”. Name may include file path.

- LeftDepthMapName

Class: String

Description: A name of left depth map. File format is YUV 4:2:0 video. File name has to be the form that “LeftDepthMapName.yuv”. Name may include file path.

- RightDepthMapName

Class: String

Description: A name of right depth map. File format is YUV 4:2:0 video. Name could include file path. File name has to be the form that “RightDepthMapName.yuv”.

- OutputVirtualViewIamgeName

Class: String

Description: A file name of virtual view YUV 4:2:0 video. Output file name is “OutputVirtualViewImageName.yuv”.

  - Camera parameter file

Intrinsic and extrinsic parameters of real and virtual cameras are gathered in a text file. Then, each camera has to be named for identifying them. For example, cameras which correspond to the view00, view01, and view02 of Nagoya dog sequence are named as param_dog00, param_dog01, and param_dog02, respectively. Figure 1 shows an example of camera parameter file.

Fig.1 An example of camera parameter file.

  - View setting file

The configuration file (.cfg) is assigned from the command line argument. Table 1 shows the parameters which are included in view setting file. In this version of software, Z_near and Z_far values of left view and Z_near and Z_far values of right view can be assigned independently.

Table 1 Input parameters

| Order | Parameter Name |
| --- | --- |
| 1 | DepthType |
| 2 | WidthOfViewImage |
| 3 | HeightOfViewImage |
| 4 | NearestDepthValueOfLeftView |
| 5 | FarthestDepthValueOfLeftView |
| 6 | NearestDepthValueOfRightView |
| 7 | FarthestDepthValueOfRightView |
| 8 | TotalNumberOfFrames |
| 9 | Camera parameter file name |
| 10 | LeftCameraName |
| 11 | VirtualCameraName |
| 12 | RightCameraName |
| 13 | LeftViewImageName (This may include file path) |
| 14 | RightViewImageName (This may include file path) |
| 15 | LeftDepthMapName (This may include file path) |
| 16 | RightDepthMapName (This may include file path) |
| 17 | OutputVirtualViewImageName (This may include file path) |

Figure 2 shows an example of view setting file. Width and height of image are 1280 (pixel) and 960 (pixel). Left, virtual, and right camera names are param_dog038, param_dog039, and param_dog041, respectively. Znear and Zfar values are those from left and right cameras. Total number of frames is 300. YUV input video file names are dog038.yuv and dog041.yuv. These names may include file paths. Depth map video file names are depth_dog038.yuv and depth_dog041.yuv. Name of output virtual view video is dog_virtual039.yuv.

Fig.2 An example of view setting file.

- Usage

A configuration file has to be read from the first command line argument.

Example:  Command line call for view synthesis.

viewsynthesis.exe viewsynthesis.cfg

Note:

For Microsoft ballet sequence, the following three lines in warping.cpp file have to be comment out.

cvexConvertCameraParam(inMat_L_cpy, exMat_L_cpy, height);

cvexConvertCameraParam(inMat_R_cpy, exMat_R_cpy, height);

cvexConvertCameraParam(inMat_V_cpy, exMat_V_cpy, height);

References

[1]M. Tanimoto, T. Fujii and K. Suzuki, “Multi-view depth map of Rena and Akko & Kayo”, ISO/IEC JTC1/SC29/WG11, M14888, October 2007.

[2]M. Tanimoto, T. Fujii and K. Suzuki, “Experiment of view synthesis using multi-view depth”, ISO/IEC JTC1/SC29/WG11, M14889, October 2007.

[3] M. Tanimoto, T. Fujii and K. Suzuki, “Available Technologies for FTV”, ISO/IEC JTC1/SC29/WG11, M15088, January 2008.

[4] M. Tanimoto, T. Fujii and K. Suzuki, “Improvement of Depth Map Estimation and View Synthesis”, ISO/IEC JTC1/SC29/WG11, M15090, January 2008.

[5] M. Tanimoto, T. Fujii, K. Suzuki, N. Fukushima, and Y. Mori, “Reference Softwares for Depth Estimation and View Synthesis”, ISO/IEC JTC1/SC29/WG11, M15377, April 2008.

Appendix: Depth Computation

Depth type has to be selected in the proposed depth estimation software and view synthesis software as mentioned in the previous section. The difference between Type I depth and Type II depth is explained here.

Depth estimation software computes depth from camera or the origin of 3D space. The following equation expresses the relation among depth Z, nearest clipping plane Znear, and farthest clipping plane Zfar from the origin of 3D space or camera.

Z=1.0v255.0⋅1.0Znear−1.0Zfar+1.0Zfar                                                   (1)

Here v is an intensity of depth map. It is noted that the values Z, Znear, and Zfar have to be either all positive values or all negative values. Otherwise, this equation can not be applied to depth values. Equation (1) is always applicable to depth from camera because Z, Znear, and Zfar from a camera are always positive values. On the other hand, all depths from the origin of 3D space are not always either positive or negative. Figure A.1 shows an example of this case. If all depths from the origin of 3D space are always either positive or negative, Eq.(1) is available. Otherwise, the following relation has to be used:

Z=Tz+1.0v255.0⋅1.0Znear−1.0Zfar+1.0Zfar.                                            (2)

Here Z is depth from the origin of 3D space but Znear and Zfar are depth from camera. Parameter Tz is the z component of translation vector. Figure A.1 shows the relation between the direction of camera and the 3D coordinate system of Nagoya Univ. Akko&Kayo and Rena sequences.

Fig. A.1 Camera arrangement and 3D coordinate system of Nagoya method

Cameras are arranged in 1D parallel and the camera array is almost parallel to the x axis of 3D coordinate system. On the other hand, the direction of cameras is opposite to the direction of the z axis of 3D coordinate system. Therefore, depth value is positive or negative.

Now, let f and l be camera focal length and camera interval, respectively. Let d and Δd be disparity and camera offset. If the following three relations between disparity and depth from the origin of 3D space are substituted to Eq.(1)

Z=Tz+f⋅ld+ΔdZnear=Tz+f⋅ldmax+ΔdZfar=Tz+f⋅ldmin+Δd,

then intensity v of depth map is expressed as

v=255⋅Tz∗dmax+Δd+f⋅l∗d−dminTz∗d+Δd+f⋅l∗dmax−dmin.                                    (3)

This is Type II method. On the other hand, if the following three relations are substituted to Eq.(2)

Z=f⋅ld+ΔdZnear=f⋅ldmax+ΔdZfar=f⋅ldmin+Δd,

then intensity v of depth map is expressed as

v=255⋅d−dmindmax−dmin. 　　　　                                     (4)

This is Type I method. Therefore, depth estimation software is equipped these two depth computation method.

*********************************************************************